The Basics of TELENET

Part I

This Bulletin is the first in a series to cover the general procedures of the

major data networks:

Telenet

Tymnet

Autonet

Arpanet

[More to be added]

BACKGROUND

----------

Telenet connects many large computers to itself through dedicated telephone

lines, each of these 'host computers' is assigned a node address(ie.:NPAxx).

Telenet is an international data network, connecting to computers around the

world.See the International telenet bulletin for more on this.

CONNECTING

----------

Telenet is probably the most 'user friendly' network of the four listed.  A

normal logon looks like this:  [NPA=Area code,xx=node address] You hit <CR>

<CR> [2 Returns] Telenet respnds with:TELENET NPAxx

TERMINAL=(Here you type your terminal identifier) (See the bulletin section for

the list)

@

The '@' is Telenet's promt to you to go ahead.

Things to do on telenet

-----------------------

To connect to a node type:

@C NPAxx

Telenet will atempt to connect to the computer at the node given.

A few of the things that Telenet will say are:

NPAxx CONNECTED (You are connected to a computer)

ILLEGAL ADDRESS (Not a working node)

NPAxx NOT REACHABLE   (The computer at that node is 'DOWN')

NPAxx REFUSED  COLLECT CONNECTION(Needs a paid ID [More later]

NPAxx REJECTING (Computer is 'UP' but not available)

NPAxx NOT RESPONDING (The computer at that node is 'DOWN')

These are most of the things that Telenet will tell you

MORE THINGS TO DO

-----------------

Typing an @ while in a host computer will return you to Telenet You will

still be connected to the other computer, you may:  D (to disconnect from that

computer) or TAPE (Unknown, possibly records your actions so that) (you may

look at them later)

If you type either of the above, while not connected to a node telenet will

respond with NOT CONNECTED

MISC.

-----

RESET (Returns Telenet to the beggining,you must start again (with the

<CR><CR>)

SET (Unknown)

MAIL (To connect to GTE Telemail) It will ask for User?  and Password?

NOTES

-----

In addition to the normal area codes in NPAxx there are also other NPA's used

in Telenet, 909 and 311, for instance.

Telenet hangs up after three disconnections from host computers, this

unfortunately is quite a PAIN in the ___ There is no way to get around this as

far as I can tell.

The ID command in Telenet is to Identify users to the host computers.  You

must type ID (your ID#) (your password).  If you just type ID, telenet will

tell you that your ID is cleared, which means nothing.  To recieve a paid user

ID on telenet call them at:  Telenet customer service:800-336-0437 800-572-

0408(In Virginia)

HACKING

-------

Telenet is very convinient for hackers as it connects many computers to your

terminal, without having to find and dial many numbers.  Start your Telehacking

by picking an areacode and then trying all the nodes in that NPA.  You will no

doubt find many interesting computers 'to work on'.

Here are instructions for using TELENET.  There are some very basic things,

which most people already know, and some other things, which even the most

dedicated hackers have probably never even heard of.  This includes things such

as international access, etc.  Well, have fun.

THE TELENET CONNECTION

INTERNATIONAL ADDRESSING/INTERNATIONAL ACCESS PROCEDURES

I.  TELENET INTERNATIONAL ADDRESS FORMAT

---------------------------- Data Network

|                       Identification Code (DNIC)

|

|

|

|      ------------------- Area Code

|      |

|      |

|      |    ------------- DTE Address

|      |    |

|      |    |      ----- Port Address

|      |    |      |

|      |    |      |

DDDD    AAA  HHHHH   PP

|

----- Optional Subaddress

Field for Packet Mode DTE

Example:  Telenet        International

-------         -------------

212 141         3110 21200141

909  84         3110 90900084

II.   ACCESS TO OVERSEAS PUBLIC DATA NETWORKS

1.  Turn on the terminal and coupler.

2.  Dial the nearest Telenet access number (See Telenet Public Dial listing).

When you hear a high-pitched tone, place the telephone

receiver in the coupler.

For Data Sets (Bell 103 or 113 type), depress the data button.

3.  Type Two carriage returns (CR).

4.  Telenet will give you a port identification number and ask you

to identify your terminal type in the two or four

character id for your terminal followed by a carriage

return (CR) or type carriage return (CR).

(EX.)   TELENET

202 DL9

TERMINAL = AJ63(CR)

5.  After Telenet prompts with a '@' type 'ID', skip a space (SP)

and type your password followed by a carriage return.  (Contact

your GTE Telenet Representative to obtain a required caller

paid ID.)

(EX.)   @ID(SP);INTL(CR)

Type in your password.

(EX.)   PASSWORD = 123456(CR)

6.  After Telenet prompts with an @, type a C. skip a space and

type the network address of the computer you wish to access,

followed by a carriage return (CR).

(EX.)   @C(SP)023411234567890(CR)

Note:  Your International address will follow a format such as:

020801234567890 for France/Transpac

023421234567890 for United Kingdom/British Telecom

026241234567890 for Germany/Datex-P

7.  Telenet will respond with a connection message.  You are now

ready to begin your conversation with the host computer.

(EX.)   (ADDRESS)CONNECTED

8.  To disconnect from your computer, log off as usual.  Telenet

will send you a disconnected message.

(EX.)   (ADDRESS)DISCONNECTED

Hang up to disconnect from Telenet.

(CR) = Carriage return

(SP) = Space

Downloaded From P-80 Systems 304-744-2253